Assignment 4: Exploring Instruction-Level Parallelism

Subject: Computer Architecture and Design

Name: Sai Sravani Pillarisetty

University Name: University of the Cumberlands

Professor Name: Sean Perryman

Date: 03/01/2026

PART 1

Introduction (What ILP Is and Why It Matters)

The capacity of a processor to carry out several distinct instructions concurrently within a single thread is known as instruction-level parallelism, or ILP. Since frequency scaling and core-count scaling have reached a plateau, modern high-performance processors mostly rely on ILP to boost single-thread performance.

According to recent studies that examine dynamic dependency graphs (DDGs) of general-purpose workloads, the upper bound on ILP is remarkably near to the IPC capabilities of the most advanced processors available today. This implies that there may soon be fundamental architectural restrictions to the continuous reliance on ILP scalability. ILP is still essential for improving performance in modern CPU designs, despite this.

Evolution of ILP (Milestones + Paradigm Shifts)

Pipelining → Superscalar → Out-of-Order → Speculation → SMT → Heterogeneity

ILP's development over time displays growing architectural sophistication.

Superscalar Execution

In order to enable the issuance of several instructions each cycle, superscalar processors expanded the issue width. Processors used to withstand about four instructions per cycle, but since 2015, vendors have improved dynamic scheduling and widened pipelines to steadily boost sustainable IPC.

Out-of-Order Execution

Instead of rigidly adhering to program order, out-of-order (OoO) execution allowed instructions to run as soon as their operands were ready. In order to maintain high ILP while controlling complexity and conflict, effective issue queue design became essential.

Register Renaming and ISA-Level Innovation

In order to get rid of misleading dependencies, traditional OoO systems rely on hardware register renaming. By rewriting ISA semantics, the Clockhands proposal presents a rename-free instruction set architecture that does away with traditional rename reasoning. This maintains the advantages of OoO execution while lowering hardware complexity and verification expense.

Speculation

Speculative execution techniques—such as branch prediction, memory-dependence prediction, and value prediction—allow processors to break dependency chains temporarily. Dependency graph analysis shows that perfect branch prediction and infinite window scaling can significantly increase theoretical ILP. However, practical predictor accuracy limits the achievable performance gains.

Simultaneous Multithreading (SMT)

SMT exploits thread-level parallelism to improve core resource utilization. However, resource sharing introduces intra-core interference. Research on SMT scheduling for ARM processors demonstrates that intelligent pairing of synergistic threads can reduce turnaround time by up to 55% in mixed workloads compared to default scheduling.

Heterogeneity

Architectural evolution is progressively taking into account specialist accelerators and heterogeneous cores to augment general-purpose OoO cores when ILP scaling slows.

Core ILP Concepts

Parallelism detection/exploitation

Dynamic dependency graphs are used to calculate ILP, which is calculated by dividing the total number of dynamic instructions by the longest dependency path (Chadwick et al., 2025).

Processors take advantage of ILP by:

Large issue pipelines

Renaming a register

Scheduling that is dynamic (issue queues)

Speculation

Sharing of SMT resources

Limitations: data deps, scontrol deps, resource limits

Data dependencies create unavoidable serialization.

Control dependencies reduce effective window utilization when branch predictions fail.

Resource limits (ROB size, issue queue entries, cache bandwidth) constrain parallel execution.

Rename logic and wakeup/select circuitry increase complexity and energy cost (Jeong et al., 2022; Koizumi et al., 2023).

Metrics: throughput, latency, power; tradeoffs

Across the selected papers, evaluation metrics include:

IPC (Instructions per Cycle)

Turnaround Time (TT)

Throughput

Fairness

Energy efficiency

Stall breakdown (frontend vs backend)

Tradeoffs include:

Wider issue vs diminishing IPC returns (Chadwick et al., 2025)

Rename hardware complexity vs ISA redesign (Koizumi et al., 2023)

Scheduling flexibility vs energy efficiency (Jeong et al., 2022)

SMT utilization vs interference (Navarro et al., 2024)

Contemporary Challenges

Complexity, diminishing returns, power/thermal limits

Beyond certain thresholds, Chadwick et al. (2025) show that increasing window size and issue width results in diminishing ILP advantages. Significant execution time is still present in low-parallelism zones, even with incredibly big windows.

Wakeup/select logic is an example of an out-of-order scheduling scheme that scales badly in terms of hardware complexity and energy consumption (Jeong et al., 2022). Additionally, wider rename logic raises front-end scalability challenges and power consumption (Koizumi et al., 2023).

Wide-issue designs and aggressive window enlargement are further restricted by power and thermal limitations.

Verification/design cost, security implications of speculation

Verification complexity is further increased by rename logic and dynamic scheduling. By minimizing renaming hardware, Clockhands specifically addresses this issue (Koizumi et al., 2023).

Speculative execution increases architectural complexity and the expense of design verification by including recovery structures and rollback procedures. Speculation enhances ILP, but it must be carefully controlled to strike a balance between implementation overhead and performance (Chadwick et al., 2025).

How researchers address challenges

Paper 1: The Future of Instruction-Level Parallelism (Chadwick et al., 2025)

Problem

Traditional width and window scaling may exceed inherent program ILP.

Method

Dynamic dependency graph modeling under multiple scenarios (finite window, infinite window, oracle branch prediction).

Tradeoffs

Extreme scaling yields theoretical gains but high hardware cost with diminishing practical benefit.

Evidence

SPEC CPU 2017 analysis shows realistic configurations nearing inherent ILP limits.

Critique

Provides strong modeling insight but does not implement a concrete hardware solution.

Paper 2: Clockhands (Koizumi et al., 2023)

Problem

Register renaming limits scalability and increases power consumption.

Method

Rename-free ISA using grouped registers (“hands”).

Tradeoffs

Reduces hardware complexity but requires ISA and compiler changes.

Evidence

7.4% energy reduction at 8-fetch width; 24.4% at 16-fetch width with comparable performance.

Critique

Adoption requires ecosystem changes; evaluated via simulation and FPGA.

Paper 3: Reconstructing Out-of-Order Issue Queue (Jeong et al., 2022)

Problem

Wakeup/select logic in OoO issue queues consumes high energy.

Method

Ballerino architecture combining cascaded and clustered in-order issue queues.

Tradeoffs

Balances performance with energy efficiency; structural redesign increases design complexity.

Evidence

Performance comparable to 8-wide OoO core; ~20% core-wide energy efficiency improvement.

Critique

Focuses on IQ domain; broader pipeline integration costs not deeply explored.

Paper 4: SYNPA (Navarro et al., 2024)

Problem

SMT thread interference degrades performance under default scheduling.

Method

Regression-based synergy prediction using dispatch-stage counters.

Tradeoffs

Adds runtime monitoring overhead for improved fairness and throughput.

Evidence

36% average turnaround time improvement; up to 55% in mixed workloads.

Critique

Evaluated on ARM SMT2 and HPC workloads; generalization requires adaptation.

Cross-Paper Synthesis

Where they agree

Brute-force pipeline widening is insufficient for sustainable ILP growth, as all authors acknowledge. They place a strong emphasis on intelligent management, efficiency, and decreased complexity.

Where they differ

Chadwick et al. (2025): modeling and limits analysis

Koizumi et al. (2023): ISA-level redesign

Jeong et al. (2022): microarchitectural restructuring

Navarro et al. (2024): runtime scheduling optimization

Trends that emerge

Efficiency-first ILP design

Reduced hardware complexity

Smarter scheduling policies

Recognition of fundamental ILP ceilings

Future Directions

ILP scaling in the future will probably include:

OoO cores combined with specialized accelerators in heterogeneous architectures

Scheduling and performance modeling using machine learning

Simplifying at the ISA level

Dynamic scheduling that considers energy

Improvements in selective speculation

Future designs will integrate clever resource management and architectural simplification rather than just expanding width.

Conclusion

Processor performance is still largely dependent on instruction-level parallelism. Modeling, however, reveals intrinsic ILP limitations in actual workloads. Instead than using brute-force pipeline expansion, sustainable performance growth necessitates architectural innovation, less structural complexity, and more intelligent scheduling procedures. Promising avenues for further ILP research include energy-efficient issue queues, rename-free ISA designs, and synergy-aware SMT scheduling.

References (APA 7)

Chadwick, A. W., Erdős, M., Bora, U., Bhosale, A., Lytton, B., Guo, Y., Cooper, R., Gabrielli, G., & Jones, T. M. (2025). The future of instruction-level parallelism (ILP). IEEE International Symposium on Performance Analysis of Systems and Software (ISPASS).

Jeong, I., Lee, J., Yoon, M. K., & Ro, W. W. (2022). Reconstructing out-of-order issue queue. Proceedings of the 55th IEEE/ACM International Symposium on Microarchitecture (MICRO).

Koizumi, T., Shioya, R., Sugita, S., Amano, T., Degawa, Y., Kadomoto, J., Irie, H., & Sakai, S. (2023). Clockhands: Rename-free instruction set architecture for out-of-order processors. Proceedings of the 56th IEEE/ACM International Symposium on Microarchitecture (MICRO).

Navarro, M., Feliu, J., Petit, S., Gómez, M. E., & Sahuquillo, J. (2024). SYNPA: SMT performance analysis and allocation of threads to cores in ARM processors. IEEE International Parallel and Distributed Processing Symposium (IPDPS).